Wireless extension tools porting guide

export PATH=$PATH:/toolchain/bin/

(A) wpa_supplicant_hostapd-0.8_rtw_20110524.zip

(1) wpa_supplicant

(1-1)unzip wpa_supplicant_hostapd-0.8_rtw_20110524.zip

(1-2)cd wpa_supplicant_hostapd-0.8\wpa_supplicant

(1-3) vim Makefile

added:

CC = mipsel-linux-gcc

(1-4)make

(1-5)copy "wpa_supplicant" and "wpa_cli" to target platform

(1-6)copy "wpa1.conf" file to target platform

(1-7)./wpa_supplicant -iwlan0 -Dwext -c wpa1.conf -dd &

(1-8)./wps_cli (for wps in Interactive mode)

(1-9) > help

> wps_pbc

> wps_pin any 12345678

(2) hostapd

(1-1)unzip wpa_supplicant_hostapd-0.8_rtw_20110524.zip

(1-2)cd wpa_supplicant_hostapd-0.8\hostapd

(1-3) vim Makefile

added:

CC = mipsel-linux-gcc

(1-4)make

(1-5)copy "hostapd" and "hostapd_cli" to target platform

(1-6)copy "rtl_hostapd.conf" file to target platform

(1-7)start hostapd daemon:

./hostapd rtl_hostapd.conf -B

(B)wpa_supplicant-0.6.9.tar.gz

(1)unzip wpa_supplicant-0.6.9_wps_patch_20100201_1.zip

(2)cd wpa_supplicant-0.6.9\wpa_supplicant

(3)cp defconfig .config

(4)vim .config

line 184:

#CONFIG_WPS=y (-)

CONFIG_WPS=y (+)

(5)vim Makefile

added:

CC = mipsel-linux-gcc

(6)make

(C)wireless_tools.29.tar.gz

(1)tar zxvf wireless_tools.29.tar.gz

(2)cd wireless_tools.29

(3)vim Makefile

line 12:

CC = mipsel-linux-gcc

line 14:

AR = mipsel-linux-ar

RANLIB = ranlib

(4)make

(5)copy "iwconfig" and "iwlist" to target platform \usr\bin

(6)copy "iwlib.so" to target platform \lib or \usr\lib